TOYOTA
	ELECTRONIC
	PARTS
	CATALOG
	SYSTEM

SET-UP MANUAL (for TYPE-1)

FOR WINDOWS VERSION

JAN 1,2000
TOYOTA MOTOR CORPORATION

Contents

Ⅰ. SYSTEM REQUIREMENTS	3

Ⅱ.	 INITIAL INSTALLATION FOR THE EPC SYSTEM USAGE  	5

Ⅲ.	 THE EPC PROGRAM UPDATE	30

Ⅳ.	 PROBLEM DETERMINATION	33

Ⅴ.	 CUSTOMIZATION OF THE EPC SYSTEM 	36

Ⅵ.	 EPC FUNCTIONS TO BE LOCALIZED BY USERS	37
	(1) Price File Setup 	38
	(2) Locally Supplied Parts Setup	41
	(3) Group Code Setup 	44
	(4) Screen File Setup	49

Ⅶ.	 SYSTEM LIMITS	52

Ⅷ.	 APPENDIX			APPENDIX 1	53
	APPENDIX 2  	57
	APPENDIX 3	58
	APPENDIX 4	66

Ⅸ.	 Guide for EPC Setup for Windows NT Server & 98 Clients	70

IBM is a trademark of International Business Machines Corporation.

Windows98, Windows NT, Windows and Microsoft Excel are trademarks of Microsoft Corporation.

Ⅰ.  SYSTEM REQUIREMENTS

1. STANDALONE

| ITEM | MODEL | NOTE |
| --- | --- | --- |
| PC | IBM AT-BUS Compatible Model | ◆Supporting Windows 98 Memory : 32MB or more H/D : *1 Display : 1024×768 Pixels or higher-resolution |
| CD Drive | Not specified | ◆Supporting Windows 98 |
| Printer | Laser Printer Only | ◆Supporting Windows 98 (Not supported Line Printer) |
| Operating S/W | Windows 98 |  |

*1) Hard Disk Capacity for Standalone EPC (①＋②＋③)

| USE |  | NECESSARY H/D CAPACITY |
| --- | --- | --- |
| ①OS operation |  | ◆Suitable |
| ②EPC operation | CD Data from CD Drive | ◆50MB per 1 Spec. |
|  | CD Data in H/D (※ 640MB per 1CD) | a) Initial: 50MB or more b) Catalogue CD: 2GB per 1 Spec. c) Frame No. CD: 640MB |
| ③Local data (Price/Locally Supplied P/No.) |  | ◆Suitable |

(Example)  CD Data for Europe spec.(A1,A2,B1) and Frame No. on Hard disk

①: Suitable
			②: a)+b)+c) = (More than 50MB)+2GB+640MB
			           = More than 2690MB
			③: Suitable
			①+②+③=Necessary H/D capacity

2. MULTI-TERMINAL

| ITEM |  | MODEL | NOTE |
| --- | --- | --- | --- |
| PC | Server | IBM AT-BUS Compatible Model | ◆Supporting Windows NT4.0 Memory : 64MB or more H/D : *2 Display : 1024×768 Pixels or higher-resolution |
|  | Client | IBM AT-BUS Compatible Model | ◆Supporting Windows 98 Memory : 32MB or more H/D : *3 Display : 1024×768 Pixels or higher-resolution |
| CD Drive | Server | Not specified | ◆Server must equip at least one CD drive supporting Windows NT4.0 |
|  | Client | Not specified | ◆Not necessary for EPC |
| Printer | Server | Laser Printer Only | ◆Supporting Windows NT4.0 |
|  | Client | Not specified | ◆Not necessary for EPC |
| Operating S/W | Server | Windows NT4.0 |  |
|  | Client | Windows 98 |  |

| USE |  |  | NECESSARY H/D CAPACITY |
| --- | --- | --- | --- |
| Server | ①OS operation (Windows NT4.0) |  | ◆Suitable |
|  | ②EPC operation | CD Data from CD Drive | ◆70MB per 1 Spec. |
|  |  | CD Data in H/D (※ 640MB per 1CD) | a) Initial: 70MB or more b) Catalogue CD: 2GB per 1 Spec. c) Frame No. CD: 640MB |
|  | ③Local data (Price/Locally Supplied P/No.) |  | ◆Suitable |
| Client | ④OS operation (Windows 98) |  | ◆Suitable |
|  | ⑤EPC operation |  | ◆50MB or more |

*2) Hard disk capacity for Server PC to run EPC (①+②+③)

*3) Hard disk capacity for Client PC to run EPC (④+⑤)

Ⅱ. INITIAL INSTALLATION FOR THE EPC SYSTEM USAGE

◇To install the TOYOTA Electronic Parts Catalog System program for the first time, please follow
  next steps.

1.	Start up Windows.

2.	Insert EPC Program CD-ROM in a CD-ROM drive.

3.	Click the Start button in the taskbar, and then select Run.

4.	In the Run dialog box, type the drive letter, followed by a colon(:) and a backslash(＼),
	and the letter win98＼setup.

For example:   (CD drive is ‘d’ drive)
		     d:＼win98＼setup

5.	Initial installation screen flow.

5-1.	If you start installation for the first time, you see this screen(①).
	If not, you see that screen(③).

蝀

To continue installation, click OK button and you see this screen(②).
	  To exit installation, click Cancel button.

蝁

To restart Windows, click Yes Button.
	  Then Windows updates some system files, you will can run setup again.(See page 5 step 1)

When the initial installation is started, you see this screen.

蝂

To start installation, click OK button. To exit installation, click Exit Setup button.

5-2.	If you have already installed EPC program in the specified drive, you see this screen.

If you want to re-install the new programs, click Yes button. If not, click No button.

5-3.	You see this screen.

蝂

蝃

蝁

蝀

①	To select hard disk drive letter for EPC application program file, click the arrow next 
    to the look in box, and then click the drive.

②	To select hard disk drive letter for EPC data (CD Set-up data which means index data of 
    catalogue CDs), click the arrow next to the look in box, and then click the drive.

③	To exit installation, click Exit button.

④	To go to next screen, click Next button.

5-4.	You see this screen

蝀

蝁

蝂

蝄

蝃

①	To select the quantity of CD drives you use, click the arrows to how many CD drives 
	you use for the EPC system.

②	To select the CD drive letter, click the arrow next to the look in box, and then click 
	the drive.

③	To exit installation, click Exit button.

④	To go to previous screen, click Back button.

⑤	To go to next screen, click Next button.

5-5.	You see this screen.

蝀

蝁

蝃

蝂

①	If you use the EPC in Client/Server environment, click YES button. If not, click NO button.

②	To exit installation, click Exit button.

③	To go to previous screen, click Back button.

④	After clicking Finish button, the system begins copying EPC program from a PGM CD to your	harddisk you specified. (It may take 5 minutes at most)
	If PGM installation is completed, you can see the following screen.

Click OK button, closing the screen.

5-6.	If you click YES button at 5-5, you see this screen.

蝀

蝁

蝃

蝂

①	In case of a Server PC, click YES button. In case of a Client PC, click NO button.

②	To exit installation, click Exit button.

③	To go to previous screen, click Back button.

④	After clicking Finish button, the system begins copying EPC program from a PGM CD to your	harddisk you specified. (It may take 5 minutes at most)
	If PGM installation is completed, you can see the following screen.

Click OK button, closing the screen.

5-7.	If you click NO button at 5-6, you see this screen.

蝀

蝁

蝃

蝂

①	To select the Server’s drive for EPC program, click the arrow next, and then select the drive.

NOTICE: Please confirm the drive letter for program installation at Server in advance.

②	To exit installation, click Exit button.

③	To go to previous screen, click Back button.

④	After clicking Finish button, the system begin copying EPC program from a PGM CD to your	harddisk you specified. (It may take 5 minutes at most)
	If PGM installation is completed, you can see the following screen.

Click OK button, closing the screen.

6.	You have to change display settings.
	Using Control Panel, you can change your screen resolution.

6-1.	Click the right mouse button on the desktop, then appears the menu,
	and point to Properties in the menu.

6-2.	You see Control Panel dialog box. Double-click a Display icon.

6-3.	You see Display Properties dialog box.
	① Select Settings tab.
	② Drag the slider to change screen resolution(1024 by 768 pixels higher-resolution).
	③ Press Advanced button.

蝁

蝀

蝂

蝅

6-4.	You see Adapter Properties(e.g. ATI Rage/Rage II PCI) dialog box.
	④ In the Font size box, select the Small Fonts.
	⑤ Click OK button.(See the Windows Help for more details.)
	⑥ Click OK button on the Display Properties dialog box.

蝃

蝄

7.	To Set up a printer you use with EPC system.
	If you want to change printer for EPC system, refer the following methods(from 7-1 to 7-3).

7-1.	Click the Start button, point to Settings, and then select Printers.

7-2.	Click a printer you use with EPC.

7-3.	On the File menu, click Set As Default.

8.	To start EPC system, click the Start button, point to Programs,
	point to EPC for Windows98, and click EPC for Windows98.

9. “Initial Setup” on “MAIN MENU”

Before you start P/No. translation in EPC, first of all, you must set the appropriate environment 
on “Initial Setup” (Menu # 8), and execute “CD Setup” (Menu # 7) after Initial setup.

10.	Set the appropriate environment.

蝀

蝁

蝂

蝄

蝃

蝅

蝆

After some items are changed, press F10 key for updating.
And return to “MAIN MENU” by pressing either F3 or F4 key.

①Another CD area version
  Once you have finished “CD Setup”, available CD areas are shown in the “Current CD Area Version”
  column. (See①).  Before you actually use the EPC system, therefore, choose just one area by clicking
  button.  When you want to use another area of CD, all you have to do is change the area in this
  column and you don’t need to set up CD again.

②Data Language

You can choose one catalogue data language by clicking button.  Figure title, Part Name Code,
  Abbreviation, Characteristic Code & Name data are displayed by the selected language.

③Screen Language
  You can choose one the screen language by clicking button.
  If you want to use local language, select Local and setup Screen File. (See Page 49)

④Latest CD Setup language
  You can see the last Data Language.

⑤Drive Letter Select to use Files
  You can select the harddisk drive which you want to setup Price File and Locally Supplied Parts File.
  The client can use the server’s data, therefore it is not necessary to setup these data in the client.
  However, please select the drive letter setup by server in listbox.

⑥Another Interface Applications for Part Number Translation Results
  You can register the software which you want to start after making save file.
  Press Refer button in the field, open file window appears to enable you to select the software, and 
  after pressing Exec button the software will be registered.

If you press Intf button on Part Number translation Results screen, the registered software are 
  displayed, then you can make save file and start the software. (See page 60 for save file format)

⑦Another Interface Applications for Save File List
  You can register the software which you want to start after opening save file.
  Press Refer button in the field, open file window appears to enable you to select the software, and 
  after pressing Exec button the software will be registered.

If you press Intf button on save file data screen, the registered software are displayed, then you can
  make save file and start the software. (See page 60 for save file format)

11.	Next, please select “CD Set-up” from “CD Setup” (Menu # 7) of “MAIN MENU”.

12.	You see this screen.

Choose your setup procedure from the following cases.

Case 1: Multi-terminal System

1.	CD setup at server PC	See page 22
	2.	To share server’s CD data by client PC 	See page 26
	3.	To delete the setup CD data by server PC	See page 27
	4.	To cancel to share server’s CD data by client PC	See page 28

Case 2: Standalone System

1.	CD setup at standalone PC	See page 22
	2.	To delete the setup CD data by standalone PC	See page 27

CD Setup at Server PC / at Standalone PC

If you setup CD for the first time, proceed to step 1. (See Figure 1)

In case of update of Catalog CDs or Frame No. CDs, proceed to step 2. (See Figure 3.2)

Figure 1

1. Check the CD areas which you want to setup. (See ① in Figure 2)
   (e.g. If you want to setup the Catalog CD for European, check the “European area version” )

Note: If you don’t check, the CD setup isn’t executed in the areas.

Figure 2

蝀

2. If you change the drive for CD setup data, click the next to the look in box, and then click drive.
   (See ① in Figure 3-1)

Figure 3.1

蝀

If some data was setup, you can see the status as follows.

If you don’t check in this area, the setup CD data will neither be updated
                   nor deleted.

蝀

蝁

蝂

蝃

Figure 3.2

① CD ID (A1,A2 or B1)
② CD-ROM	: To read CD data from CD-ROM drive
   Hard Disk	: To store all CD data in Hard Disk
③ CD-ROM creation time at TMC (MM/DD/YY)
④ CD-ROM Status (inserted or blank)

3. In the “CD SETUP” screen , press ADD button in the appropriate area. (See Figure 3.1)
  The “Change Status” dialog box appears to enable you to select CD IDs to be setup or updated.

Figure 4.1  (for Catalog CD)

蝀

蝁

Figure 4.2  (for Frame No. CD)

蝁

蝀

4. From the “Change Status” dialog box, select CD IDs  (See ① in Figure 4.1 or 4.2)

5. Select “CD-ROM/Hard Disk” radio button. (See ② in Figure 4.1 or 4.2)

6. If you continue the job, press OK button. To exit the job, press Cancel button.

7. You can see this screen.

Figure 5

8. To start CD setup, press F10 key.
   To clear setting information, press F11 key.

9. System requires to insert the CD by the following dialog box.
   Please insert an appropriate CD according to the message and press Retry button.
   If you want to stop this job, press Cancel button.  (See Figure 6)

Figure 6

[Target Drive = ##] ##: drive letter that you will insert Catalog CD or Frame No. CD
	   (ANY: anywhere drive)
[Target CD = *** ##]***: kind of CD
	     Catalog CD:	Eur=Europe, Gen=General, U.S.A=USA, JPN=Japan
	     Frame No. CD: JPN=Japan, OSV=Overseas
	  ##: CD ID(For example: A1, A2 or B1)

10. If CD setup job is completed, it returns to “CD SETUP” screen.

# To Share Server’s CD Data by Client PC

The client uses the server’s CD data, therefore, it is not necessary to setup CD data in the client.
However, please make preparation to share the server’s CD data as follows:

Figure 8

蝀

蝁

1. Check the CD areas which you want to use by the client PC (See ① in Figure 8)
   You can see the drive letters setup by server in listbox. (See ② in Figure 8)

2. To start this action, press F10 key. To clear this setting, press F11 key.
   To cancel this action and return to “CD SETUP” screen, press F3 key.
   To cancel this action and return to “MAIN MENU” screen, press F4 key.

3. If this job is completed, it returns to “CD SETUP” screen.

# To Delete the Setup CD data by Server PC / Standalone PC

Figure 7

蝀

蝁

蝂

To delete the CD set up data per area

1. Check the CD areas which you want to delete (See ① in Figure 7)

Note: If you don’t check, CD setup data will not be deleted.

2. Select CD IDs you want to delete in the list box. (See ② in Figure 7)

Note: If you delete all the CD IDs, CD setup data will be deleted per area,

And if you delete some CD IDs, CD setup data will be deleted per CDs.

3. And press DEL button (See ③ in Figure 7)

4. To start this action, press F10 key. To clear the setting, press F11 key.
  To cancel this action and return to “CD SETUP” screen, press F3 key.
  To cancel this action and return to “MAIN MENU” screen, press F4 key.

5. If this job is completed, it returns to “CD SETUP” screen.

# To Cancel to Share Server’s CD data by Client PC

Figure 9

蝀

1. Clear the check for the CD areas which you want to cancel (See ① in Figure 9)

2. To start this action, press F10 key. To clear this setting, press F11 key.
   To cancel this action and return to “CD SETUP” screen, press F3 key.
   To cancel this action and return to “MAIN MENU” screen, press F4 key.

3. If this job is completed, it returns to “CD SETUP” screen.

14.	Lexus catalogue data is protected by the password.  If you need Lexus catalogue data, click
	“LEXUS Dealer Check” and input the password provided by TMC.
	The password is changed by CD data area and CD issue month.
	It isn’t necessary to input the password at Client PC.

Ⅳ.  THE EPC PROGRAM UPDATE

This procedure is to update EPC program.

1.	Click the Start button, point to Settings, and then select Control Panel.

2.	Click a Add/Remove Programs.

3.	Select EPC for Windows98, click the Add/Remove button.

4.	You see this screen.

If you want to completely remove EPC for Windows98 and all of its components, click YES button.
 If not, click NO button.

5.	You see “Shared Component” screen, click Keep button.

6.	Complete remove components, you can see the following screen.

Click OK button to close the screen.

7.	Start installation. (See page 5 for more details)

Ⅳ. PROBLEM DETERMINATION

(1) If System Error Occurs

- Make a hardcopy of the screen

- ①In case of an error with the EPC program, please make a hardcopy with F6 key
②In case of a system error with Windows O/S, please make a hardcopy with Print Screen key.
③If you can not print out, please write down the error codes and messages.

- Inform TMC of your situation with the above information explaining your operations until you face  the problem if you need TMC support.

- Exit EPC program
When EPC system is hung up, press the Ctrl+Alt+Del keys.
		-You can see 'Close Program'.
		-Point to "EPC for Windows98" of it.
		-Click End Task button.
To restart EPC system, click the Start button, then point to Programs,
point to EPC for Windows98, and click EPC for Windows98. (See page 15 for more details)

(2) How to Solve Printing Problems

CASE 1: You cannot print in EPC at all.

1. Click the Start button, point to Settings, then click Printers.

2. Use the right mouse button to click the printer you want to use, then click Properties
   on the menu.

3. Click the Graphics tab, then click None as the Dithering.

## CASE 2:  You can print out text data but cannot not image data in EPC.

1. Click the Start button, point to Settings, then click Printers.

2. Use the right mouse button to click the printer you want to use, 
    then click Properties on the menu .

3. Click the Details tab, then click the Spool Settings button.(①)

蝀

4. Select “EMF” as the Spool data format(②), and press OK button.

蝁

Ⅴ. CUSTOMIZATION OF THE EPC SYSTEM

TOYOTA Electronic Parts Catalog System supports four languages, English, French, German, and
Spanish officially. However, the screen text, message and help files can be altered according to local
requirements.

- Screen Text: As for the setup procedure for Screen File Setup, please see page 49.

(2) Operation and Error Messages

1. Edit GCDROM09.MSG by use of an edit program
  (GCDROM09.MSG is located on X:＼TMCEPCW2 directory.  The client PC can use the directory
  located in Server PC, therefore, it is not necessary to change it by client PC itself.)

2. Basic rule for the message file is as follows;
  ①Each message must start with three-digit message ID.
  ②You can't add a new message ID.
  ③Each message can be composed to two line of 71 characters including the leading message ID.
     It can also be composed of just one line. However, the following message must be one line
     length. (Message No.999)
  ④Message No. 100 - 799 are guidance messages.
     Message No. 800 - 999 are error messages. (See Appendix 1 for more details.)

3. After editing, select "Initial Setup" from “MAIN MENU” to change the Screen Language to local.
     (See page 16 for more details)

(3) Help

1. Edit GCXXXX09.HLP by use of an edit program.
  (GCXXXX09.HLP is located on X:＼TMCEPCW2 directory. The client PC can use the directory
  located in Server PC, therefore, it is not necessary to change it by client PC itself.)

2. Basic rule for the help file is as follows;.
 ①Each line should not be more than 84 characters long.
 ②Each file contains help messages which are displayed by pressing F1.
    (See Appendix 2 for more details.)

3. After editing, select "Initial Setup" from “MAIN MENU” to change the Screen Language to local.
  (See page 16 for more details)

NOTE:

If you want to change Screen Text, Operation and Error Massages or Help, you must use
GCXXXX09.xxx(xxx: SCR, MSG, HLP).  To use GCXXXX09.xxx, change Screen Language to local
in Initial Setup screen. (See page 16 for more details)

Ⅵ. EPC FUNCTIONS TO BE LOCALIZED BY USERS

The EPC system has functions to be localized by users for local unique data such as price and locally supplied parts etc.  These data must be prepared and setup by users according to the following methods.

(Menu for EPC functions to be localized by users)

(1) Price File Setup

(2) Locally Supplied Parts Setup

(3) Group Setup

(4) Screen File Setup

- Price File Setup

1. Data Preparation

The local price file prepared in diskette or harddisk with the following file name can be used
	to display the price data with P/No. in EPC.  
	As for the price file data layout, please refer to the Appendix 3.

| Price Data | File Name |  |
| --- | --- | --- |
|  | ①Text File Format (Same as price file of DOS EPC) | ②CSV Format (XXX is a serial #) |
| 1 kind of price | PRICE | PRICEXXX |
| 3 kinds of price | PRICE3 | PRICE3XXX |

①Text File Format: 
	The text file must be prepared either by a user program or by an edit program which allows
	editing in binary mode. In this file, please do not use CR(carriage return), LF(line feed), and
	EOF(end of file) characters.

- CSV Format:
CSV file format is generally supported by MICROSOFT EXCEL etc. In this format, you can easily add new data to the file or up-date necessary data in the file.

2.  File Setup Procedure

① From the “MAIN MENU” screen, press “CD Setup” button.

② From the “CD SETUP” screen, press “Price File Setup” button.
    The “PRICE FILE SETUP” dialog box appears. (See Figure 1)

Figure 1

③Press Refer button, and then you see Open dialog box.

Figure 2

④To setup the price file from floppy disk, insert the floppy disk to the drive.

⑤Select the kind of price in the Files of type listbox. (See Figure 3)
     "1 Kind of Price"	 One kind of price file (Old Format)
     "1 Kind of Price(CSV)"	 One kind of price file (New Format)
     "3 Kinds of Price"	 3 kinds of price(Retail, Trade, Cost) file (Old Format)
     "3 Kinds of Price(CSV)"	 3 kinds of price(Retail, Trade, Cost) file (New Format)

Figure 3

⑥To setup from other folders except for the default, select appropriate one in the folder box.

⑦Select the name of price file from the dialog box, and then press Open button.

⑧You can see the selected price File Name and Kind Of Price in the PRICE FILE SETUP dialog box.

Figure 4

⑨To start price file setup, press Exec button. To cancel setting up, press Clear button.

⑩When price file has been already setup, the question dialog box displays, as shown in Figure 5.
     If you want to overwrite price file, press Yes button. If not, press No button.

Figure 5

⑪Remove the floppy disk from the drive, and insert the floppy disk of next price file to the drive.
     Repeat steps④to ⑩for the price file setup.

Figure 6

⑫Complete set up all price file, press Exit button, and then close this screen.

- Locally Supplied Parts Setup

1. Data Preparation

The local supplied parts data prepared in floppy disk or hard disk with the following file name
	can be used to display them as P/No. translation results in EPC.  
	As for the data layout, please refer to the Appendix 3.

| File Name | Data Contents | File Format |
| --- | --- | --- |
| LOCALP | One TMC (Japan sourced) P/No. vs One local supplied P/No. | Text File Format (Same as the format of DOS EPC) |
| LOCALP1 | One TMC (Japan sourced) P/No. vs Local supplied P/No(s) | CSV Format |
| LOCALP2 | Local unique P/Nos. under TMC PNC | CSV Format |
| LOCALP3 | Local unique P/Nos. & illustrations | CSV Format |

2. File Setup Procedure

①From the “MAIN MENU” screen, press “CD Setup” button.

②From the “CD SETUP” screen, press “Locally Supplied Parts Setup” button.
    The “LOCALLY SUPPLIED PARTS SETUP” dialog box appears. (See Figure 1)

Figure 1

③To set up locally supplied parts file from the floppy disk, insert the floppy disk to the drive.

④Press Refer button, and then you see Open dialog box. (See Figure 2)

Figure 2

⑤Select the kind of local in the files of list box. (See Figure 3)
     "TMC Part Number"         ---> TMC Part Number:Local Part Number=1:1  (Old Format)
     "TMC Part Number   (CSV)" ---> TMC Part Number:Local Part Number=1:N
     "TMC Part Name Code(CSV)" ---> (Catalog Code+PNC of TMC):Local Part Number=1:N
     "Local Figure         (CSV)" ---> (Catalog Code+Local Fig. No.):Local Part Number=1:N

Figure 3

⑥To set up the file from other folders except for the default, select appropriate folder in the folder box.

⑦Select the name of locally supplied parts file from the dialog box, and then press Open button.

⑧To start installation, press Exec button. To exit installation, press Exit button.
     To cancel setting regarding this File Name and File Type, press Clear button. (See Figure 4)

Figure 4

⑨When the locally supplied parts file has been already setup, the question window displays.
    (See Figure 5)
    If you want to setup additional data, press Yes button and repeat steps⑤to⑨.
    If not, press No button and go to step⑩.

Figure 5

⑩If you select Local Figure (CSV) by step 5, then you can see name of locally supplied parts image
     file you will setup in the listbox. (See Figure 6)

Figure 6

⑪If necessary, use the Drive box to specify a drive for locally supplied parts image file.

⑫If necessary, use the Directory box to specify a directory for locally supplied parts image file.

⑬The name of the image file appears in the listbox, then press Image Setup button.
     The installer sets up the image file, and you can see the asterisk (*) with the file in the listbox.
     The asterisk (*) means that the image file is setup completely.

Note: If you setup only text file for local unique P/Nos without the image file, only text data

will be displayed.

⑭The all locally supplied parts image file sets up completely, press Exit button to close this dialog
   box.

## (3) Group Code Setup

A group of Part Numbers of frequent demand (e.g. V-belt, oil seal), or frequent repair (e.g. bumper)
can be recorded by assigning a number with the brief explanation in description column.
The setup procedure is as follows:

1.From the “MAIN MENU” screen, press “CD Setup” button.

2.From the “CD SETUP” screen, press “Group Setup” button.
    The “GROUP CODE SETUP” screen appears. (See Figure 1)

Figure 1

3. Please input the group code referring explanations in the below.

蝁

蝀

①Type group code directly into the box, and press Enter key. (e.g.  007)

②If the group code of①already exists, the description is displayed in the box, and furthermore,
   PNCs for the group are also displayed in the listbox.

③By use of List button, it is possible to see the list all group codes registered.
   (See Figure 3 of next page)

④By use of Del button, it is possible to delete a group code.

Firstly input a existing group code, and press Del button.

(Group Code List)

蝂

Double-click the group code in the listbox or 
select the group code in the listbox, and press Enter key.

4. After input Description, and press Enter key. (See ① in Figure 4)

Figure 2

蝃

5. To input PNC for the group, and press Enter key. (See ① in Figure 5)
    (1) If the PNC exists, the description is displayed in the listbox.
    (2) If not, error message is displayed.
    (3) To delete a PNC, input it in PNC # column, and press Del button. (See①&② in Figure 5)

Figure 3

Figure 4

蝀

6. When the contents of the group code are finalized and you are ready to register, press PF10 key
    to save the group code.  If you press PF3 or PF4 key, no group code is saved, and then close this
    screen.

## (4) Screen File Setup

It is possible to display EPC screen text by local language if you replace the text with local language

according to the following procedure.

1. From the “MAIN MENU” screen, press “CD Setup” button.

2. From “CD SETUP” screen, press “Screen File Setup” button.
    The “SCREEN FILE SETUP” screen appears. (See Figure 1)

Figure 5

3. To select Screen Name, click the arrow next to the look in box.

4. To change screen text, double-click screen text in the list, 
     and then you can see “SCREEN TEXT EDIT” dialog box. (See Figure 2)

蝁

5. Input text in local language in the column of “Screen Text New”, press OK button, and change
  the screen text.  If you press Cancel button, dialog box is closed and the screen text isn’t changed.

6. After you replace the screen text with local language, you can see the asterisk(*) with 
    it in the listbox.  (See Figure 3)
    If you want to save the screen text, press PF10 key, and returns “CD SETUP” screen.
    If you want to cancel this job, press PF3 or PF4 key.

蝀

Ⅶ. SYSTEM LIMITS

Please consider the following system limits in TOYOTA Electronic Parts Catalog System.

| Limit is set on ; | Limit |
| --- | --- |
| Figure Number selected on Illustrated Index | 100 |
| Figure Number selected on Keyword Search | 100 |
| Part Name Codes selected on Entire and Detail Figure | 1,000 |
| Part Name Codes specified on PNC Input (Bypassing Figure) | 1,000 |
| Part Name as results of Part Number Translation (Match-off condition) | 3,000 |
| Save File saved on the Part Number Translation Results | 9,999 |
| Group Codes | 1,000 |
| Part Name Code Belonging to one Group Code | 100 |
| Part Numbers on Local Price File | 16,000,000 |
| Part Numbers on Locally Supplied Parts File | 10,000 |

Ⅷ. APPENDIX

APPENDIX 1

Message List

100	Vehicle Input, Part Number Translation Results
101	Catalog Code Select
102	Vehicle Model Select
103	Search Route
104	Search Route
105	Part Number Application to Models
106	Search Route
107	Search Route
108	Search Route
109	Search Route, Part Number Application to Models
110	Keyword Search
111	Part Number Application to Models
112	PNC Input
113	Save File List
114	New Parts List
115	Main Menu
116	Vehicle Input, Vehicle Model Select
117	Illustrated Index
118	Figure(Entire)
119	Figure(Detail)
120	Figure(Detail), Part Number Translation Results,
	Part Number Application to Models, New Parts List, Part Number List
121	PNC Input
122	Part Number Translation Results
123	List Price
124	List Price
125	Part Number List
126	Part Number List
127	CD Setup
140	Part Number Application to Models, Part Number List 
141	New Parts List
150	CD Setup
151	CD Setup
152	Initial Setup
153	CD Setup
154	Group Code Setup
155	Group Code Setup

156	Group Code Setup
157	Group Search
158	Part Number Translation Results
159	Screen File Setup
160	Part Number Translation Results
161	Save File Data
162	Search Route
163	CD Setup
164	CD Setup
165	CD Setup
166	CD Setup
167	CD Setup
168	CD Setup
169	CD Setup
300	<not used>
301	<not used>
400	<not used>
401	CD Setup
402	Initial Setup
403 	CD Setup
404	CD Setup
405	Group Code Setup
406	CD Setup
407	CD Setup
408	CD Setup
409	<not used>
600	CD Setup
601	CD Setup
700	<for all the screens>
701	Part Number Translation Results
702	Part Number Translation Results, Save File Data
703	Save File List
704	Save File List
705	CD Setup
706	CD Setup
707	Figure(Entire, Detail), Part Number Translation Results,
	Part Number Application to Models, Part Number List(only TYPE-1/TYPE-2)
	New Parts List, List Price, Group Setup
708	CD Setup
709	CD Setup
710	Keyword Search
711	New Parts List
712	Initial Setup

713	CD Setup
714	Group Code Setup
715	Group Code Setup
716	CD Setup
717	Group Code Setup
718	<not used>
719	Group Code Setup, Group Search
801	<for all the screens>
802	<for all the screens>
810	<for all the screens>
811	Keyword Search, Illustrated Index
812	Figure(Entire), Figure(Detail)
813	Part Number Translation Results
814	Part Number Translation Results
815	Save File List
816	Keyword Search
817	CD Setup
818	New Parts List
819	Group Code Setup
820	Vehicle Input
821	CD Setup
822	Initial Setup
823	CD Setup
824	<not used>
825	<not used>
826	<not used>
828	<not used>
830	Search Route
831	Illustrated Index
832	Figure(Entire), Figure(Detail)
833	PNC Input
834	PNC Input
835	Figure(Entire), Figure(Detail), PNC Input
836	Part Number Translation Results
837	CD Setup
838	Figure(Detail), Part Number Translation Results, Part Number Application to Models,
	New Parts List, Part Number List(only TYPE-1/TYPE-2)
839	Part Number Translation Results, Main Menu, List Price
840	<for all the screens>
841	<for all the screens>
842	<for all the screens>

843	Vehicle Input
844	Part Number Application to Models, List Price
845	New Parts List
846	CD Setup
847	CD Setup
848	CD Setup
849	Search Route
850	CD Setup
851	Group Code Setup
852	Save File List
853	Part Number Translation Results
854	Vehicle Input
855	<not used>
860	Vehicle Input, Part Number Translation Results
861	Illustrated Index
862	Figure(Entire)
863	Figure(Entire)
864	Illustrated Index
865	<for all the screens>
866	Search Route
867	Search Route
868	Search Route
869	<for all the screens>
870	Initial Setup
871	CD Setup
872	New Parts List
873	New Parts List
874	New Parts List
875	Group Code Setup
876	Main Menu
877	Search Route
880	<for all the screens>
881	<for all the screens>
882	New Parts List
883	New Parts List
890	<when Toyota Parts Catalog System started>
891	Main Menu
892	<not used>
900	<not used>
910	CD Setup(only TYPE-1/TYPE-2)
917	CD Setup(only TYPE-1/TYPE-2)
999	<for all the screens>

APPENDIX 2

Relationship Between the File Name and the Screen

GCGYOM09.HLP	Main Menu
GCVJIN09.HLP	Vehicle Input
GCFSEL09.HLP	Save File Select
GCVCAT09.HLP	Catalog Code Select
GCVKEI09.HLP	Vehicle Model Select
GCKROT09.HLP	Search Route
GCKKWD09.HLP	Keyword Search
GCKPNC09.HLP	PNC Input
GCEMOK09.HLP	Illustrated Index
GCBUNZ09.HLP	Figure(Entire)
GCBUNK09.HLP	Figure(Detail)
GCBUSZ09.HLP	Figure(Entire) Reference
GCBUSK09.HLP	Figure(Detail) Reference
GCPNHO09.HLP	Part Number Translation Results
GCPRIC09.HLP	List Price (from Part Number Translation)
GCSIUP09.HLP	Initial Setup
GCSTUP09.HLP	CD Setup
GCTEKI09.HLP	Part Number Application to Model
GCNEWP09.HLP	New Parts List
GCDPRC09.HLP	List Price (from Main Menu)
GCKGRL09.HLP	Group Search
GCKGRP09.HLP	Group Part Name Code
GCSTGP09.HLP	Group Code Setup
GCFSAV09.HLP	Save File List
GCFSAD09.HLP	Save File Data
GCSSCR09.HLP	Screen File Setup
GCHINB09.HLP	Part Number List
GCLPEM09.HLP	Local Figure Number List
GCLPPS09.HLP	Detail Illustration of Local Parts

APPENDIX 3

(1) SAVE FILE

①SAVE FILE MASTER ("GCIFMST.SAV")

File Format :

| No | Starting column | Length | Content | Note |
| --- | --- | --- | --- | --- |
| 1 | 0 | 2 | Number of Save Files | Equals the number of entries with Use Flag ‘1’ in this master file |
| 2 | 2 | 2 | Next File Number | The number assigned to next created save file as file id |
| 3 | 4 | 12 | Reserved | Not used currently |
| 4 | 16 | 1 | Use Flag | x’01’ - When the corresponding file exist ‘D’ - When the corresponding file doesn’t exist |
| 5 | 17 | 3 | Reserved | Not use currently |
| 6 | 20 | 6 | Saved Date |  |
| 7 | 26 | 6 | Saved Time |  |
| 8 ¦ ¦ N | 32 |  | Repetition of field No.4 - 7 |  |

The file “GCIF0001.SAV” corresponds to the field on No.4 - 7 of this master file and “GCIF0002.SAV”
to field No.8 - 11.

②SAVE FILE ("GCIFxxxx.SAV") ...this can be applied to temporary file)
	-xxxx is a sequential number starting with ‘0001’.

File Format :Registration

| No | Starting column | Length | Content | Note |
| --- | --- | --- | --- | --- |
| 1 | 0 | 17 | VIN |  |
| 2 | 17 | 15 | Model |  |
| 3 | 32 | 6 | Catalog Code |  |
| 4 | 38 | 6 | Production Date | YYYYMMDD |
| 5 | 44 | 7 | Frame Model |  |
| 6 | 51 | 1 | Frame Hyphen |  |
| 7 | 52 | 7 | Frame No |  |
| 8 | 59 | 3 | Color Code |  |
| 9 | 62 | 4 | Trim Code |  |
| 10 | 66 | 30 | Notes |  |
| 11 | 96 | 3 | The number of saved part number |  |
| 12 | 99 | 12 | Part Number |  |
| 13 | 111 | 2 | Quantity |  |
| 14 ¦ N | 113 |  | Repetition of field No.12 - 13 |  |

③SAVE FILE ("xxxxxxxx.TXT") ...you can make this file in unique name and folder

File Format : (TEXT format)

| No | Starting column | Length | Content | Note |
| --- | --- | --- | --- | --- |
| 1 | 0 | 17 | VIN |  |
| 2 | 17 | 15 | Model |  |
| 3 | 32 | 6 | Catalog Code |  |
| 4 | 38 | 6 | Production Date | YYYYMMDD |
| 5 | 44 | 7 | Frame Model |  |
| 6 | 51 | 1 | Frame Hyphen |  |
| 7 | 52 | 7 | Frame No |  |
| 8 | 59 | 3 | Color Code |  |
| 9 | 62 | 4 | Trim Code |  |
| 10 | 66 | 30 | Notes |  |
| 11 | 96 | 3 | The number of saved part number |  |
| 12 | 99 | 12 | Part Number |  |
| 13 | 111 | 2 | Quantity |  |
| 14 ¦ N | 113 |  | Repetition of field No.12 - 13 |  |

④SAVE FILE ("xxxxxxxx.CSV") ...you can make this file in unique name and folder

File Format : (CSV format)

| No | Length | Content | Note |
| --- | --- | --- | --- |
| 1 | 17 | VIN |  |
| 2 | Max 15 | Model |  |
| 3 | 6 | Catalog Code |  |
| 4 | 6 | Production Date | YYYYMMDD |
| 5 | Max 7 | Frame Model |  |
| 6 | 7 | Frame No |  |
| 7 | 3 | Color Code |  |
| 8 | 4 | Trim Code |  |
| 9 | Max 30 | Notes |  |
| 10 | 3 | The number of saved part number |  |
| 11 | Max 12 | Part Number |  |
| 12 | 2 | Quantity |  |
| 13 ¦ N |  | Repetition of field No.11 - 12 |  |

(2) Vehicle Information File

①Unique No. or Name (Text file format)

| Starting column | Length | Content | Note |
| --- | --- | --- | --- |
| 0 | 17 | VIN/VDS |  |
| 17 | 15 | Model |  |
| 32 | 6 | Catalog Code |  |
| 38 | 6 | Production Date | YYYYMM |
| 44 | 7 | Basic Model Number | Frame Numbers |
| 51 | 1 | Hyphen “-“ | Frame Numbers |
| 52 | 7 | Serial Number | Frame Numbers |
| 59 | 3 | Color Code |  |
| 62 | 4 | Trim Code |  |
| 66 | 30 | Notes |  |

NOTE:
	-Text file must not have CR(carriage return), LF(line feed), and EOF(end of file) characters.

②Unique No. or Name (CSV format)

| Length | Content | Note |
| --- | --- | --- |
| 17 | VIN/VDS |  |
| Max 15 | Model |  |
| 6 | Catalog Code |  |
| 6 | Production Date | YYYYMM |
| Max 7 | Basic Model Number | Frame Numbers |
| 7 | Serial Number | Frame Numbers |
| 3 | Color Code |  |
| 4 | Trim Code |  |
| Max 30 | Notes |  |

NOTE:
	-Each data must be divided by comma(,), but it is not necessary to add double quotation mark(”””).
	-If the data include comma(,), the data muse be added double quotation mark(“””).
	-CSV format is generally supported by Microsoft Excel etc.

(3) Local Price File

①One kind of price (Text file format)
  File Name : PRICE

| Starting column | Length | Content | Note |
| --- | --- | --- | --- |
| 0 | 12 | Part Number |  |
| 12 | 8 | Price | including “,” or “.” |
| 20 | 10 | Remarks |  |
| 30 | 20 | Part Name |  |

NOTE:
	-“,” in ‘Price’ should be specified by three digits.
	-“,” in ‘Price’ means a decimal point.
	-‘Price’(including ‘List Price’) field is allowed only number except “,” and “.”.

②One kind of price (CSV file format)
   File Name: PRICExxx.CSV ... xxx is a sequential number starting with ‘001’.

| Length | Content | Note |
| --- | --- | --- |
| Between 10 and 12 | Part Number |  |
| Max 8 | Price | including “,” or “.” |
| Max 10 | Remarks |  |
| Max 20 | Part Name |  |

NOTE:
	-“,” in ‘Price’ should be specified by three digits.
	-“,” in ‘Price’ means a decimal point.
	-‘Price’(including ‘List Price’) field is allowed only number except “,” and “.”.
	-Each data must be divided by comma(,), but it is not necessary to add double quotation mark(“”).
	-If the include comma(,), the data must be added double quotation mark(“”).
	-You must make sorted price data by the key of Part Number.
	-CSV file format is generally supported by Microsoft Excel etc.

③Three kinds of price (Text file format)
  File Name:  PRICE3

| Starting column | Length | Content | Note |
| --- | --- | --- | --- |
| 0 | 12 | Part Number |  |
| 12 | 8 | List Price | including “,” or “.” |
| 20 | 8 | Trade Price | including “,” or “.” |
| 28 | 8 | Cost Price | including “,” or “.” |
| 36 | 10 | Remarks |  |
| 45 | 20 | Part Name |  |

NOTE:
	-“,” in ‘Price’ should be specified by three digits.
	-“,” in ‘Price’ means a decimal point.
	-‘Price’(including ‘List Price’, ‘Trade Price’ and ‘Cost Price’) field is allowed only number except
	 “,” and “.”.

④Three kinds of price (CSV file format)
  File Name:  PRIC3xxx.CSV.. .xxx is a sequential number starting with ‘001’.

| Length | Content | Note |
| --- | --- | --- |
| Between 10 and 12 | Part Number |  |
| Max 8 | List Price | including “,” or “.” |
| Max 8 | Trade Price | including “,” or “.” |
| Max 8 | Cost Price | including “,” or “.” |
| Max 10 | Remarks |  |
| Max 20 | Part Name |  |

NOTE:
	-“,” in ‘Price’ should be specified by three digits.
	-“,” in ‘Price’ means a decimal point.
	-‘Price’(including ‘List Price’, ‘Trade Price’ and ‘Cost Price’) field is allowed only number except
	 “,” and “.”.
	-Each data must be divided by comma(,), but it is not necessary to add double quotation mark(“”).
	-If the include comma(,), the data must be added double quotation mark(“”).
	-You must make sorted price data by the key of Part Number.
	-CSV file format is generally supported by Microsoft Excel etc.

(4) Locally Supplied Parts File

 ①One TMC P/No. vs. One Locally supplied P/No. 
   File Name:  LOCALP(Text file format, it is the same format as EPC DOS version)

| Starting column | Length | Content | Note |
| --- | --- | --- | --- |
| 0 | 12 | Japan source part number | part number to be replaced by locally supplied part number |
| 12 | 12 | Locally Supplied Part Number |  |
| 24 | 20 | Remarks |  |

②One TMC P/No. vs. Locally supplied P/No(s)
  File Name:  LOCALP1(CSV file format)

| Length | Content | Note |
| --- | --- | --- |
| Between 10 and 12 | Japan Source Part Number |  |
| Between 10 and 12 | Locally Supplied Part Number |  |
| 4 | Production Date(From) | YYMM |
| 4 | Production Date(To) | YYMM |
| Max 30 | Remarks |  |

NOTE:
	-Each data must be divided by comma(,), but it is not necessary to add double quotation mark(“”).
	-If the include comma(,), the data must be added double quotation mark(“”).
	-You can make multiple data with the key of Japan Source Part Number.
	-CSV file format is generally supported by Microsoft Excel etc.

③Local unique P/Nos. under TMC PNC.
  File Name:  LOCALP2(CSV file format)

| Length | Content | Note |
| --- | --- | --- |
| 6 | Catalog Code |  |
| Max 6 | TMC part name code |  |
| Between 10 and 12 | Local Unique Part Number |  |
| 4 | Production Date(From) | YYMM |
| 4 | Production Date(To) | YYMM |
| Max 30 | Remarks |  |

NOTE:
	-Each data must be divided by comma(,), but it is not necessary to add double quotation mark(“”).
	-If the include comma(,), the data must be added double quotation mark(“”).
	-You can make multiple data with the key of Catalog Code and TMC part name code.
	-CSV file format is generally supported by Microsoft Excel etc..

④Local unique illustrations and P/Nos. 
  File Name:  LOCALP3(CSV file format)

| Length | Content | Note |
| --- | --- | --- |
| 6 | Catalog Code |  |
| 4 | Fig No |  |
| Max 30 | Fig Name |  |
| Max 12 | Image File Name | file name of Local unique illustration |
| Between 10 and 12 | Local Unique Part Number |  |
| 4 | Production Date(From) | YYMM |
| 4 | Production Date(To) | YYMM |
| Max 30 | Part Name |  |
| Max 30 | Remarks |  |

NOTE:
	-Each data must be divided by comma(,), but it is not necessary to add double quotation mark(“”).
	-If the include comma(,), the data must be added double quotation mark(“”).
	-You can make multiple data with the key of Catalog Code, Fig No, Fig Name and 
	 Image File Name.
	-Image data must be made according to the bitmap format (BMP) within the size of 643 dot ×
	 599 dot
	-CSV file format is generally supported by Microsoft Excel etc..

APPENDIX 4

1. Files Used by TOYOTA Electronic Parts Catalog System

(1) When the TOYOTA Electronic Parts Catalog System is initially installed the following files are
   copied from the CD.

to X:＼TMCEPCW2

GCDROM.EXE	load module
GCALL.DLL	=
GCVJIN.DLL	=
GCVCAT.DLL	=
GCKROT.DLL	=
GCEMOK.DLL	=
GCLPPS.DLL	=
GCBUNZ.DLL	=
GCPNHO.DLL	=
GCTEKI.DLL	=
GCDPRC.DLL	=
GCNEWP.DLL	=
GCFSAV.DLL	=
GCHINB.DLL	=
GCSTUP.DLL	=
GCSTGP.DLL	=
GCSIUP.DLL	=

GCDROM.SYS	system environment file
GCDROM0x.SCR	screen text(x is for 1, 2, 3, 4, and 9)

only to server's X:＼TMCEPCW2

GCDROM0x.MSG	operation and error message(x is for 1, 2, 3, 4, and 9)
GCGYOM0x.HLP	help(x is for 1, 2, 3, 4, and 9)
GCVJIN0x.HLP	=
GCVCAT0x.HLP	=
GCVKEI0x.HLP	=
GCKROT0x.HLP	=
GCKPNC0x.HLP	=
GCKGRL0x.HLP	=
GCKGRP0x.HLP	=
GCKKWD0x.HLP	=
GCEMOK0x.HLP	=
GCBUNZ0x.HLP	=
GCBUNK0x.HLP	=
GCBUSZ0x.HLP	=
GCBUSK0x.HLP	=
GCPNHO0x.HLP	=
GCPRIC0x.HLP	=
GCTEKI0x.HLP	=
GCDPRC0x.HLP	=
GCNEWP0x.HLP	=
GCFSAV0x.HLP	=
GCFSAD0x.HLP	=
GCSTUP0x.HLP	=
GCSTGP0x.HLP	=
GCSSCR0x.HLP	=
GCSIUP0x.HLP	=
GCHINB0x.HLP	=
GCFSEL0x.HLP	=
GCLPEM0x.HLP	=
GCLPPS0x.HLP	=

(2) After the completion of CD Setup, the following files are created.

(Catalog CD Data)

GCROOTMF.MST
GCXXXXXX.MST
XXXXXXXX.DAT
XXXXXXXX.IA
XXXXXXXX.IS
XXXXXXXX.IK

(Frame CD Data)

GCFRNOMF.MST
FRAMENO.IA

(3) After the completion of Price File Setup, the following files are created.

to X:＼TMCMSTW2

(in the case of including one kind of price)
	KAKAKU.D
	KAKAKU.IA
	PRICE.MST

(in the case of including three kinds of price)
	PRICE3.D
	PRICE3.IA
	PRICE.MST

(4) After the completion of Locally Supplied Parts Setup, the following files are created.

to X:＼TMCMSTW2

(in the case of TOYOTA part number)
	LOCALP1.D
	LOCALP1.IA
(in the case of TOYOTA part name code)
	LOCALP2.D
	LOCALP2.IA
(in the case of locally supplied part figure)
	LOCALP3.D
	LOCALP3.IA
	XXXXXXXX.BMP

(5) When results of the part number translation are saved, the following files are created.

to X:＼TMCMSTW2

GCIFSMST.SAV
GCIFXXXX.SAV

(6) When group codes are saved, the following files are created.

to X:＼TMCMSTW2

GCGRPMEI.D
GCGRPPNC.D

(7) When part number application to models is executed, the following files are created.

to X:＼TMCMSTW2

GCTEKI.CSV

(8) When new parts information is executed, the following files are created.

to X:＼TMCMSTW2

GCNEWP.CSV

(9) When parts number list is executed, the following files are created.

to X:＼TMCMSTW2

GCHINB.CSV

(10) Temporary files are created on the virtual disk drive during the execution of the Toyota
    Electronic Parts Catalog System.

(file name: XXXXXXXX.TMP)
Some temporary files are not erased after the completion of the Toyota Electronic Parts Catalog System, but these files can also be erased manually.

2. Environment Files

(1) GCDROM.SYS

This defines the environment for TOYOTA Electronic Parts Catalog System.

ECMST=C:＼TMCMSTW2	shows where the Current Area Catalog CD Setup files are
ECFIL=C:＼TMCEPCW2	shows where help and message files are
ECRAM= C:＼TMCTMPW2	shows where to place the temporary files
CDROM=E:,F:	shows drive letter of CD-ROM device
LANGUAGE=ENGLISH	Data language selected on Initial Setup
	English, French, German, or Spanish
MENULANG=ENGLISH	Menu language selected on Initial Setup
	English, French, German, Spanish or Local
PRICE=C:	shows drive letter where PRICE/PRICE3 files
	to be used resides, specified on Initial Setup
LOCALP=C:	shows drive letter where Locally Supplied Parts files
	to be used resides, specified on Initial Setup
CRNTAREA=EU	Used Catalog CD Data selected on Initial Setup
	English, French, German, or Spanish
ECMSTALL=EU:C, JP:D	CD Area and Drive for setupped Catalog CD Data
FRAMEWLD=C:＼TMCMSTW2	shows where the Frame CD(Over Sea Version) Setup files are
FRAMEJPN=C:＼TMCMSTW2	shows where the Frame CD(Japanese Version) Setup files are
EXPGPNHO=	external program name by Parts Number Translation Results
	specified on Initial Setup (Max 5 programs specified)
EXPGFSAD=	external program name by Save File Data
	specified on Initial Setup (a program specified)